|  | ## Compatibility List: ACDC Red, Heavy Metal, Clear Case, Green ATA PTX4 Avanti PTX4, TX4, Style, Euro Boss HT3 Red Light BFT Mitto 2, Mitto 4 Beninca TWV2 TWV4 Cenution TX4, Style, New Style DEA MIO TR2, MIO TR4, MIO TD2, MIO TD4 ECA TX4 ET Doors Red FAAC XT4433RC Fadini Astro 43-2 Gibidi Domino Series Herculift Key Sub44 Letro AR1 MHouse GTX4, TX4 Napoleon Red Smart Duo, Quattro Steel-Line ZT-07, HT3 Red Light Superlift TX4, Style, Euro |  |  | ## Standard Specs: Frequency: 433.92MHz Coding: Fixed & Rolling Code Body Type: Receiver Channels: 2 Button Colour: N/A Colour: Grey Remote Size: 90x60x20mm Range: >40 Meters |  |
| --- | --- | --- | --- | --- | --- |
| ## HOW TO CODE YOUR REMOTE Code to CHANNEL 1 Step 1: Press the CH1 LEARN button ONCE on the receiver and then release. The LED will stay solid. Step 2: Press the button required on the remote. LED will flash 5 times and turn off. Code to CHANNEL 2 Step 1: Press the CH2 LEARN button ONCE on the receiver and then release. The LED will stay solid. Step 2: Press the button required on the remote. LED will flash 5 times and turn off. |  |  | ## HOW TO DELETE ALL REMOTES Step 1: Hold LEARN button on either channel down. LED will come on for approximately 5 seconds and then go out. You have now cleared all remotes from the receiver for that channel. Repeat on other channel if required. |  |  |
|  | HOW TO WIRE A 3-WIRE SYSTEM 1 - SWITCH = CH1 N/O Terminal 2 - 12/24v POSITIVE = POS Terminal 3 - NEGATIVE = GND Terminal Link GND & CH1 COM Terminals |  |  |  |  |